FITNESS TRACKER — QUANTIFIED SELF

Human Activity Recognition & Automated Rep Counting — Technical Report

Technical Project Report  ·  George Dogo  ·  2026

# 1. Executive Summary

This report documents the design and implementation of an end-to-end Human Activity Recognition (HAR) system for gym workout analysis. The system processes raw 6-axis wrist-worn sensor data (accelerometer and gyroscope at 12.5 Hz) through a complete signal processing, feature engineering, and machine learning pipeline to automatically classify exercise types and count repetitions.

The final Random Forest classifier achieves 94.1% exercise classification accuracy across five compound exercises. The automated rep counting algorithm achieves ±1 rep accuracy on 92% of sets (MAE = 0.34 reps). The project is structured as a proper installable Python package with setup.py and a pinned Conda environment for full reproducibility.

# 2. Project Motivation & The Quantified Self

## 2.1 The Quantified Self Framework

The Quantified Self movement applies rigorous scientific measurement to personal behaviour data. In a fitness context, this transforms subjective training perception into objective, longitudinal metrics: progression rates, fatigue signatures, technique consistency, and plateau detection.

Commercial fitness trackers provide black-box implementations. This project provides a transparent, fully explainable, and customisable alternative that exposes every algorithmic decision — from filter cutoff frequency to peak detection threshold — making it a genuine research-grade tool.

## 2.2 System Objectives

- Exercise Classification — identify which of 5 compound exercises is being performed from sensor data alone

- Rep Counting — automatically count repetitions per set with commercial-tracker-grade accuracy

- Quantified Self Analytics — generate objective workout metrics enabling longitudinal performance tracking

# 3. Dataset & Sensor Specifications

## 3.1 Hardware

MetaMotion (MbientLab) wristband sensor — medical-grade inertial measurement unit (IMU) capturing 6-axis motion data.

| Specification | Value |
| --- | --- |
| Sampling Rate | 12.5 Hz (80ms between readings) |
| Accelerometer Range | ±16 g |
| Gyroscope Range | ±2000 °/sec |
| Accelerometer Resolution | 0.001 g |
| Gyroscope Resolution | 0.061 °/sec |
| Battery Life | 4+ hours continuous recording |
| Data Format | CSV per exercise set |

## 3.2 Exercise Classes & Biomechanical Signatures

| Exercise | Primary Signal Axis | Dominant Freq. (Hz) | Notes |
| --- | --- | --- | --- |
| Barbell Back Squat | acc_z (vertical) | 0.38–0.52 | Clear vertical oscillation; easiest to classify |
| Bench Press | acc_y (horizontal push) | 0.47–0.58 | Horizontal push; gyr_x differentiates from OHP |
| Barbell Deadlift | acc_z + acc_y combined | 0.30–0.42 | Slowest frequency; hip-hinge signature on gyr_y |
| Overhead Press (OHP) | acc_z (vertical press) | 0.44–0.55 | Similar to squat in vertical axis; gyr rotation key |
| Barbell Row | acc_y (horizontal pull) | 0.44–0.56 | Horizontal pull; opposite gyr_y direction vs bench |
| Rest | All axes | <0.10 | Near-zero signal; ~99% classification accuracy |

# 4. Signal Processing Pipeline

## 4.1 Outlier Detection — Chauvenet's Criterion

Raw sensor data contains implausible readings from sensor initialisation, physical contact loss, and electromagnetic interference. Chauvenet's Criterion flags observations whose probability under a fitted normal distribution falls below 1/(2N), where N is the sample count. This is a principled, distribution-aware method that adapts the rejection criterion to the dataset size — more conservative than fixed IQR thresholds.

Application: per axis, per exercise class. Flagged values are set to NaN and forward-filled to preserve temporal continuity without introducing artificial signal discontinuities.

## 4.2 Butterworth Low-Pass Filter

The Butterworth filter attenuates frequency components above the cutoff while preserving components below it with maximally flat frequency response (no passband ripple). Implementation via scipy.signal.butter() for coefficient design and scipy.signal.filtfilt() for zero-phase application.

| Parameter | Value | Justification |
| --- | --- | --- |
| Filter type | Butterworth | Maximally flat passband — no signal distortion |
| Pass type | Low-pass | Remove high-frequency noise above exercise band |
| Cutoff frequency | 1.3 Hz | Exercise motion: 0.3–1.0 Hz; margin to 1.3 Hz |
| Filter order | 5 | Steeper roll-off than lower orders; stable |
| Application | filtfilt (zero-phase) | Prevents phase delay that would misalign peaks |
| Sampling frequency | 12.5 Hz | Nyquist: 6.25 Hz; cutoff well within Nyquist range |

# 5. Feature Engineering

## 5.1 Windowing Strategy

| Parameter | Value | Justification |
| --- | --- | --- |
| Window size | 200 samples (16 seconds) | Captures ≥3 reps at typical training cadence |
| Window overlap | 50% (stride = 100 samples) | Doubles effective training set; handles offset patterns |
| Split strategy | GroupKFold by participant-set ID | Prevents same-set data appearing in train and test |
| Label assignment | Majority class in window | Handles boundary windows cleanly |

## 5.2 Feature Catalogue

| Feature Category | Features Extracted | Count |
| --- | --- | --- |
| Time-domain mean | Per-axis mean (6 axes) | 6 |
| Time-domain std | Per-axis standard deviation | 6 |
| Time-domain range | Per-axis min, max, range | 18 |
| Energy | Per-axis signal energy (Σx²/N) | 6 |
| Zero-crossing rate | Per-axis sign change frequency | 6 |
| FFT dominant frequency | Peak frequency in power spectrum | 6 |
| FFT spectral power | Total energy in frequency domain | 6 |
| FFT band power | Low-band (0–1 Hz) and high-band power | 12 |
| Temporal abstraction | Rolling stats at 3 window sizes | 36 |
| Total Features | — | 102 |

# 6. Exercise Classification

## 6.1 Model Comparison

| Model | Accuracy | F1 (macro) | Squat | Bench | Dead | OHP | Row |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Decision Tree | 85.3% | 0.82 | 0.88 | 0.81 | 0.89 | 0.74 | 0.80 |
| Random Forest (n=200) | 94.1% | 0.93 | 0.95 | 0.91 | 0.97 | 0.88 | 0.93 |
| SVM (RBF) | 91.7% | 0.90 | 0.93 | 0.88 | 0.95 | 0.86 | 0.91 |
| KNN (k=5) | 88.2% | 0.86 | 0.91 | 0.84 | 0.93 | 0.79 | 0.88 |

## 6.2 Confusion Matrix Analysis

Primary confusion occurs between Overhead Press and Bench Press (6.2% of OHP predictions misclassified as Bench, 5.1% of Bench misclassified as OHP). This is expected from a biomechanical standpoint — both exercises involve a pressing motion with similar frequency content. The primary discriminating features for these two classes are gyr_x_std (wrist rotation variance) and acc_z/acc_y ratio (vertical vs horizontal acceleration balance).

## 6.3 Top 10 Feature Importances

| Rank | Feature | Importance (Gini) | Discriminative Role |
| --- | --- | --- | --- |
| 1 | acc_z_dominant_freq | 0.108 | Exercise fundamental frequency |
| 2 | gyr_x_std | 0.094 | Wrist rotation variance (key for OHP/Bench separation) |
| 3 | acc_z_energy | 0.087 | Vertical movement intensity |
| 4 | acc_y_mean | 0.071 | Horizontal axis bias (row vs bench direction) |
| 5 | gyr_z_fft_power | 0.064 | Yaw rotation power (OHP vs squat) |
| 6 | acc_z_std | 0.059 | Vertical movement consistency |
| 7 | acc_y_energy | 0.053 | Horizontal energy (pull vs push magnitude) |
| 8 | gyr_y_dominant_freq | 0.049 | Pitch rotation frequency |
| 9 | acc_z_band_power_low | 0.047 | Low-frequency vertical energy |
| 10 | gyr_x_mean | 0.041 | Mean wrist rotation direction |

# 7. Repetition Counting

## 7.1 Algorithm Design

Repetitions are counted by detecting peaks in the low-pass filtered signal of the dominant axis for each exercise. The scipy.signal.find_peaks() function identifies local maxima meeting specified height and minimum-separation constraints.

The minimum separation parameter (distance) encodes the minimum physiologically plausible inter-rep interval — preventing double-counting of single peaks with minor secondary oscillations. This parameter was tuned per exercise type based on observed rep duration distributions.

## 7.2 Per-Exercise Configuration & Validation

| Exercise | Dominant Axis | Height Threshold | Min Distance (samples) | MAE | ±1 Accuracy |
| --- | --- | --- | --- | --- | --- |
| Squat | acc_z | 0.40 g | 10 (0.8s) | 0.31 | 94.2% |
| Bench Press | acc_y | 0.30 g | 9 (0.7s) | 0.38 | 91.3% |
| Deadlift | acc_z | 0.50 g | 15 (1.2s) | 0.25 | 96.1% |
| OHP | acc_z | 0.35 g | 10 (0.8s) | 0.42 | 89.4% |
| Barbell Row | acc_y | 0.30 g | 9 (0.7s) | 0.36 | 91.8% |
| Overall | — | — | — | 0.34 | 92.6% |

# 8. Software Engineering & Package Structure

## 8.1 Python Package Architecture

The project is structured as a proper installable Python package using setup.py. Running 'pip install -e .' in the repository root registers the /src directory as an importable module — enabling clean, path-independent imports from any notebook or script.

This architectural decision reflects production ML engineering standards: the same package structure used in professional ML codebases enables unit testing, CI/CD integration, and code reuse across projects — capabilities that notebook-only projects fundamentally lack.

## 8.2 Reproducibility

- environment.yml specifies all Conda dependencies with pinned version numbers — identical environment reproducible on any machine with 'conda env create -f environment.yml'

- setup.py enables editable install — source changes immediately reflected without reinstalling

- Sequential numbered notebooks (01 through 06) document each pipeline stage in order, enabling complete reproduction of results from raw data

# 9. Conclusion & Future Directions

This project demonstrates that a fully transparent, research-grade fitness tracking system can match commercial tracker accuracy (92.6% rep counting accuracy, 94.1% exercise classification) using only open-source tools and a clear signal processing methodology.

The most significant technical contribution is the integration of frequency-domain features (FFT-derived dominant frequency and band power) with traditional time-domain statistics — this combination is what enables the system to distinguish mechanically similar exercises (OHP vs Bench Press) that time-domain-only approaches frequently confuse.

Future work priorities: (1) Replace hand-engineered features with a 1D-CNN operating directly on filtered signal — expected accuracy improvement to ~97%; (2) Implement LSTM-based rep counter for velocity-adaptive counting; (3) Real-time inference WebSocket endpoint for live workout tracking; (4) Fatigue detection via rep duration trend analysis within sets.